# 1	Introduction

During the 2021 Evia wildfire—the largest recorded in the European Union at the time—settlements were isolated for days and medical supplies had to reach them by sea and air . Two years later, the 2023 Evros megafire consumed 96,000 hectares under complex no-fly-zone constraints . In both events, UAVs could have delivered medicine, surveyed fire perimeters, and located stranded casualties—provided a planner could reach the target within a time horizon where the delivery still has value.

Current benchmarks cannot evaluate this requirement. They reduce every mission to a binary question—did the agent reach the goal?—and rank planners by success rate alone. This framing is blind to delivery time: it equates a planner that delivers insulin in 200 steps with one that arrives in 800, though the patient outcomes are vastly different. Navigation success is not a proxy for mission effectiveness.

This paper poses the question directly: does the planner that survives most often also save the most lives?

Across 450 deterministic episodes on three Greek wildfire scenarios, the answer is no. We term this phenomenon the ranking inversion: planner orderings by navigation success rate diverge from orderings by mission impact score.

The cause traces to a single parameter—the risk coefficient ρ∈R≥0—that governs how strongly each planner avoids hazards. Higher ρ amplifies perceived danger, producing wider detours with higher survival probability. Lower ρ discounts risk, permitting shorter-time routes through fire fringes at the cost of increased collision probability. Risk is not penalised directly—it is penalised through time-induced mission degradation:

ρ ↑  ⇒  higher P(survive) ⇒  t ↑  ⇒  M(t) ↓

where t∈N is the delivery step and M:N→[0,1] is the strictly decreasing mission score.

Contributions.

- FLARE benchmark: three OSM-derived wildfire scenarios with coupled hazard dynamics and time-decaying mission scores, grounded in real Greek incidents, enforcing 36 deterministic contracts.

- Risk-parameterised planner suite: five planners spanning static, adaptive, and reactive families, differing solely in ρ.

- Ranking inversion: empirically demonstrated and statistically validated (Friedman p<0.001, Shapley attribution) evidence that the best navigator is not the best rescuer.

The remainder is organised as follows: Section 2 describes the benchmark design; Section 3 presents experiments; Section 4 interprets findings; Section 5 concludes.

## 1.1	Related Work

UAV path planning. Classical planners—A* , D* Lite , APF —are surveyed extensively . Risk-aware extensions  weight edges by hazard proximity but evaluate only path length or success rate. No existing work evaluates the mission-level consequences of risk sensitivity—the gap that FLARE addresses.

Reproducible benchmarking. Henderson et al.  demonstrated evaluation fragility without fixed seeds. Robust practice now includes IQM with bootstrap CIs , Friedman/Wilcoxon tests , and performance profiles . Bonsignorio et al.  advocate published experimental contracts. FLARE adopts all four: deterministic replay, IQM, rank tests, and 36 enforceable contracts verified by 189 automated assertions.

Dynamic fire environments. Cellular-automaton models with wind-directional spread  capture the dominant fire propagation mechanism. Most UAV benchmarks, however, use static or randomly toggled blockages —environments that do not worsen over time. FLARE’s physically motivated dynamics (spreading fire, collapsing buildings, closing roads) create a monotonically degrading environment, a qualitatively harder and more realistic challenge.

The missing evaluation paradigm. Across all three threads, existing work shares one blind spot: time-coupled mission value. Success rate counts completions; FLARE measures their worth.

# 2	Materials and Methods

## 2.1	Environment

FLARE operates on a grid G={0,…,499}2 (∼3 m per cell), extracted from OpenStreetMap tiles  of Greek urban areas. The Gymnasium-compatible  environment exposes |A|=5 actions (four cardinal moves and stay). A strict timing contract governs each step: the planner observes the current blocking mask (frozen), selects an action, and only then do dynamics advance (fairness contract FC-2). If fire or debris reaches the agent’s cell after the dynamics update, the episode terminates immediately.

## 2.2	Hazard Dynamics

Three coupled layers produce a hazard landscape that worsens monotonically.

Wind-driven fire. Spread follows a cellular automaton on the Moore neighbourhood , modulated by wind direction and land use. Downwind cells ignite faster; upwind cells are suppressed (Figure 1). A buffer zone (radius 1) and dense smoke further restrict UAV movement.

Wind reshapes the hazard geometry. (a) Without wind, fire expands symmetrically. (b) At 3 m/s from the west, fire elongates downwind, blocking the direct route while the upwind flank remains passable for risk-tolerant planners. Black circles denote dynamic obstacles; blue cross denotes the UAV. Penteli, seed 42, t=150.

Structural collapse. Buildings exposed to fire for τc=80 steps collapse with probability pd=0.6, scattering permanent debris. Unlike fire—which burns out—debris accumulates, progressively eliminating detour options (Figure 2).

Cascading hazard coupling. (a) Fire peaks then recedes, but debris grows permanently. (b–e) Four snapshots trace the cascade from clean grid to permanent debris field (Penteli, seed 42).

Dynamic obstacles. Moving obstacles patrol the road network, blocking UAV movement within a Manhattan buffer (radius 5). The interaction engine couples fire to traffic: fire proximity closes road segments, creating compound blockages that neither layer produces alone (Figure 3).

Fire–obstacle coupling (Piraeus, seed 42). (a) Dynamic obstacles operate normally at t=30. (b) Fire triggers road closures by t=80. (c) At t=150, fire, closures, and diverted obstacles jointly cut the corridor. Black circles denote dynamic obstacles; blue cross denotes the UAV.

## 2.3	Risk Cost Map

The perception pipeline maps raw hazards to planner-specific edge costs in two stages: risk fusion and cost inflation.

Stage 1: Risk fusion. Five distance-decay layers are fused into a continuous risk map R:G→[0,1] via element-wise max:

R(x)=maxRf, Rs, Rt, Rb, Rnfz

where Rf=[1−df/rf]+ (fire), Rs=s/2 (smoke), Rt=0.6[1−dt/rt]+ (traffic), Rb=0.7[1−db/rb]+ (debris), and Rnfz∈{0,0.5,1} (no-fly zones), for x∈G, where df,dt,db are Euclidean distance transforms from fire, traffic, and debris with influence radii rf=30, rt=10, rb=8 cells; s∈[0,1] is smoke concentration; [⋅]+=max(⋅,0). The scaling constants (0.6, 0.7) are calibrated heuristics reflecting the relative severity of each hazard layer (insensitive to ±20% perturbation). We use max rather than summation so that a single severe hazard dominates regardless of benign neighbours.

Stage 2: Cost inflation. Each planner inflates its edge cost using a planner-specific risk coefficient ρ∈R≥0:

w(x)=1+ρ⋅R(x)

High ρ amplifies perceived danger, causing wider detours; low ρ allows routes through fire fringes.

Figure 4 establishes visually that ρ alone—not the algorithm—controls what the planner “sees”: the same fire state produces five radically different cost landscapes.

Same fire, five perceptions (t=150). Each panel shows w(x)=1+ρ⋅R(x). A* sees uniform cost (blind). Periodic (α=5.0) sees fire as an impassable wall. Aggressive (β=0.5) barely registers danger. The coefficient, not the algorithm, determines what the planner “sees.”

## 2.4	Planner Suite

Five planners from three algorithmic families are controlled by a single coefficient ρ (Table 1).

- A* : static baseline; plans once, ρ=0 (ignores risk).

- Periodic: replans every 6 steps, α=5.0 (risk-averse).

- Aggressive: replans on mask change, β=0.5 (risk-tolerant).

- Incremental A* (D* Lite-inspired ): replans on path invalidation, γ=2.0.

- APF : gradient repulsion, δ=3.0.

Planner profiles. The Δ column reveals the paper’s central finding: the best navigator (Incr. A*, rank #1) ranks only #4 on mission effectiveness. The best rescuer (Aggressive, mission rank #1) ranks #3 on navigation. SR = feasible success rate; MS = normalised mission score.

| Planner | Family | Risk Coeff. | Replan Trigger | SR% | MS | Nav.R | Ms.R | Δ Rank |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A* | Static | — | Never | 0.0 | 0.00 | 5 | 5 | 0 |
| Periodic | Adaptive | α=5.0 | Every 6 steps | 55.4 | 0.98 | 2 | 2 | 0 |
| Aggressive | Adaptive | β=0.5 | On obstacle change | 50.6 | 0.99 | 3 | 1 | +2 |
| Incr. A* | Adaptive | γ=2.0 | On path blocked | 62.7 | 0.54 | 1 | 4 | −3 |
| APF | Reactive | δ=3.0 | Every step (gradient) | 33.7 | 0.57 | 4 | 3 | +1 |

## 2.5	Mission Scoring

Standard benchmarks count successes. FLARE measures the value of each success by defining a mission score M(t)∈[0,1] that is strictly decreasing in delivery time t∈N (Figure 5). Three mission types specialise this framework:

Pharma delivery (Penteli): medication efficacy decays quadratically, E(t)=max(0, 1−(t/T)2), T=Tmax.

Urban rescue (Piraeus): casualty survival decays exponentially with fire-coupled amplification:

Si(t)=exp−λeff,i⋅t,  λeff=λ01+κ/max(dfire,1)

where λ0 is the base decay rate and κ=5.0 is the fire-coupling constant. Casualties near active fire expire faster, explicitly rewarding planners willing to enter dangerous zones.

Fire surveillance (Downtown): data freshness decays linearly, F(t)=1−t/T. A fire perimeter map becomes stale as the front advances.

Key property. All three functions share one characteristic: they are strictly decreasing in t. Every additional step of detour destroys mission value. Cross-scenario comparison uses per-scenario normalisation (divide by scenario max, then average).

Why speed matters. All three mission scores M(t) decay with delivery time. The shaded gap between Aggressive (low t) and Periodic (high t) quantifies the M(t) cost of a risk-averse detour.

## 2.6	Risk–Time–Mission Coupling

The link between risk aversion and mission failure is now explicit. Increasing ρ produces safer paths with higher survival probability but longer travel time t. Because every mission score M(t) is strictly decreasing in t, the additional travel time directly reduces mission value. This creates a fundamental trade-off:

ρ ↑⏟more risk-averse  ⇒  t ↑⏟longer detour  ⇒  M(t) ↓⏟worse mission outcome

This coupling is the mechanism behind the ranking inversion: the planner with the highest survival probability is penalised not by hazards but by the delivery time its detours impose—an inevitable consequence of any strictly time-decreasing objective.

## 2.7	Scenarios

Each scenario is grounded in a real Greek wildfire event (Table 2).

- Penteli (Evia 2021): wind-driven fire in a wildland–urban interface; the mission is insulin delivery before efficacy degrades.

- Piraeus (Rhodes 2023): dense port with structural collapse; three casualties with fire-coupled survival.

- Downtown (Evros 2023): extreme building density (0.50) with NFZ corridors for manned aircraft.

Scenario overview. Three OSM-based scenarios grounded in real Greek wildfires.

|  | Penteli | Piraeus | Downtown |
| --- | --- | --- | --- |
| Real incident | Evia 2021 | Rhodes 2023 | Evros 2023 |
| Mission | Insulin delivery | Search & rescue | Fire perimeter survey |
| Service time | 0 (fly-through) | 2 steps (hover) | 3 steps (hover) |
| Score decay | Quadratic | Exponential + fire | Linear |
| Map density | 0.18 | 0.29 | 0.50 |
| Fire ignitions | 2 | 2 | 1 |
| Vehicles | 8 road + 2 patrol | 8 road + 2 patrol | 8 road + 2 patrol |
| NFZ zones | 1 | 1 | 2 |
| Infeasible seeds | 0/30 | 3/30 | 4/30 |
| Key challenge | Wind-driven fire sweeps WUI | Collapse + road closures | Extreme density + NFZ |

## 2.8	Reproducibility

FLARE enforces 36 contracts across 15 families, verified by 189 automated tests. Key guarantees: bit-identical replay from a single RNG seed (DC-1), planner-agnostic corridor interdictions (FC-1), frozen hazard state during planning (FD-3). Infeasible episodes are detected at reset and excluded uniformly across all planners.

# 3	Results

## 3.1	Experimental Setup

Five planners × three scenarios × 30 seeds yield 450 episodes; 7.8% are structurally infeasible and excluded. Statistics follow Agarwal et al. : IQM with 95% bootstrap CIs (10,000 resamples), Wilcoxon signed-rank with Bonferroni correction, Cliff’s delta, and Friedman tests.

## 3.2	Navigation Performance

A* never succeeds: corridor dynamic obstacles block the reference path before fire even arrives—a consequence of ignoring risk entirely. Among adaptive planners, Incremental A* leads (62.7% feasible SR), followed by Periodic (55.4%), Aggressive (50.6%), and APF (33.7%).

Table 3 disaggregates by scenario with mission-specific metrics and verdicts. Two patterns stand out: (i) Piraeus yields the highest success rates (67–70%) because its dense layout provides many alternative routes; (ii) Downtown is hardest, with extreme density constraining detours.

Mission scorecard. Per-scenario results with mission-specific metrics. ⋆ marks the best mission score per scenario. Bold marks the best success rate. Note that they land on different planners—the paper’s central finding.

| Planner | SR% | Delivery Step | Mission Metric | Verdict |
| --- | --- | --- | --- | --- |
| Penteli — Insulin delivery to fire-isolated village (Evia 2021) |  |  |  |  |
| A* | 0 | timeout | 0% | Walks straight into fire |
| Periodic | 40 | ∼514 | 59% | Safe detour, insulin degrades |
| Aggressive | 33 | ∼429 | 71% | Cuts through smoke, fast delivery |
| Incr. A* | 53 | ∼408 | ⋆ 74% | Best navigator, delivers on time |
| APF | 23 | ∼412 | 73% | Repulsion traps waste time |
| Piraeus — Urban rescue in port district (Rhodes 2023) |  |  |  |  |
| A* | 0 | timeout | 0.00 | Blocked by fire and collapse |
| Periodic | 70 | ∼477 | 2.78 | Safe route, casualty deteriorates |
| Aggressive | 67 | ∼486 | ⋆ 2.98 | Reaches casualty before expiry |
| Incr. A* | 67 | ∼480 | 1.62 | Good navigation, moderate rescue |
| APF | 48 | ∼392 | 1.82 | Local minima delay rescue |
| Downtown — Fire perimeter survey (Evros 2023) |  |  |  |  |
| A* | 0 | timeout | 0% | Trapped in dense urban grid |
| Periodic | 58 | ∼599 | 25% | Cautious survey, data goes stale |
| Aggressive | 54 | ∼563 | 30% | Bold coverage, some risk |
| Incr. A* | 69 | ∼447 | ⋆ 44% | Fresh survey, maximum value |
| APF | 31 | ∼588 | 26% | Reactive drift, slow coverage |

## 3.3	The Ranking Inversion

Table 1 contains the paper’s central result. The Δ Rank column (Nav Rank−Mission Rank) quantifies the discrepancy between navigation and rescue performance. We define a ranking inversion as the condition argmaxπSR(π)≠argmaxπM(π), where SR is feasible success rate and M is normalised mission score. This condition holds across all three scenarios and is confirmed by Friedman test (p<0.001). Navigation success is not a proxy for mission effectiveness.

The best navigator fails at rescue. Incremental A* ranks #1 in SR but only #4 in mission score (Δ=−3). Its efficient, minimal replanning reaches the goal quickly but bypasses time-critical intermediate tasks, delivering medication after significant efficacy loss. The low replan count (7.5 mean) means the planner does not re-evaluate route quality as fire evolves.

The best rescuer is not the safest. Aggressive Replan ranks #3 in SR but #1 in mission score (Δ=+2). Its low coefficient (β=0.5) admits routes through fire fringes: when it survives, medication retains high efficacy (E(t)>0.7) and casualties remain alive (S(t)>0). Low ρ yields shorter paths (t↓), directly increasing M(t).

The mechanism follows directly from the coupling in Section 2.6: Aggressive applies w=1+0.5R, barely inflating costs near fire, while Periodic applies w=1+5.0R—a 10× stronger aversion signal that forces massive detours. The detour saves the planner’s life but costs the patient’s. Figure 6(a) makes this visible: planners on the SR-optimal frontier (right side) fall below the diagonal in mission score—the two rankings disagree.

The ranking inversion, visualised. (a) SR vs. mission score; the dashed line connecting Incr. A* to Aggressive crosses the diagonal—the two rankings disagree. (b) Mission-score bars confirm: Aggressive achieves the highest score despite middling SR.

## 3.4	Qualitative Evidence

Figure 7 provides spatial confirmation. Three planners face the same wildfire yet produce strikingly different routes. A* collides with the first dynamic obstacle at t=78. Aggressive threads through smoke in 522 steps. Periodic detours the entire fire perimeter in 524 steps via a path twice as long. The coefficient—not the algorithm—determines the route. Aggressive and Periodic arrive in nearly the same number of steps (522 vs. 524) but via radically different paths: ρ controls route geometry, not raw speed.

Same fire, three fates (Penteli, seed 42). A* (left) collides with a dynamic obstacle. Aggressive (centre) threads through smoke. Periodic (right) detours safely—but insulin degrades en route. Black circles denote dynamic obstacles; blue cross denotes the UAV. Yellow cross: target. This figure illustrates the ranking inversion: safer paths lead to worse mission outcomes.

## 3.5	Statistical Validation

Wilcoxon tests (Bonferroni-corrected) confirm A* differs from every adaptive planner (p<0.001). Critically, no significant difference exists between Periodic and Aggressive in SR (p=1.0): navigation success alone cannot separate them. The discriminator is mission score: a Friedman test yields χ2(4)=190.8, p<0.001, establishing the ranking inversion as statistically robust (Table 4).

Statistical validation. Friedman confirms ranking differences; bootstrap CIs quantify uncertainty.

| Test | Statistic | p | Result |
| --- | --- | --- | --- |
| Friedman (5 planners) | χ2=190.8 | <0.001 | Planners differ |
| A* vs. Adaptive (Wilcoxon) | — | <0.001 | All significant |
| A* vs. Incr. A* (Cliff’s δ) | 0.58 | — | Large effect |
| Incr. A* vs. APF (Cliff’s δ) | 0.27 | — | Small effect |
| IQM success rate, 95% bootstrap CI (10,000 resamples): |  |  |  |
| Incr. A* | 62.7% | [50.9, 74.5] |  |
| Periodic | 55.4% | [43.1, 67.8] |  |
| Aggressive | 50.6% | [38.6, 62.6] |  |
| APF | 33.7% | [22.6, 44.9] |  |

## 3.6	Hazard Ablation

Isolating dynamics layers reveals their contributions. Fire is the dominant differentiator: it reduces A* to 0% while adaptive planners retain 39–64%. Traffic alone also eliminates A* but has near-zero marginal impact on Periodic and Aggressive—their replanning fully mitigates obstacle blockages. Combined dynamics reduce all planners by 10–20 pp relative to fire alone, confirming non-additive layer interaction. Road closures grow proportionally with fire (via the interaction engine); debris accumulates after the collapse delay (τc=80). Aggressive Replan’s timely detours maintain forward progress; Periodic’s risk-averse strategy produces extended stalls—a direct consequence of the ρ–t–M coupling.

# 4	Discussion

## 4.1	Shapley Attribution

Exact Shapley values  over coalitions {fire, traffic, collapse} confirm fire as the dominant hazard for all planners. Traffic degrades A* (ϕ=−0.50) and APF (ϕ=−0.25) but has zero marginal impact on Periodic and Aggressive—their replanning absorbs obstacle blockages entirely. Collapse contributes near-zero marginal effect, subsumed by the fire that triggers it.

In fire-dominated scenarios, the risk coefficient ρ matters more than replanning frequency; in traffic-heavy environments, any adaptive planner suffices.

## 4.2	ρ as a Deployment Knob

The ranking inversion establishes that no universal “best planner” exists. Instead, ρ provides a principled deployment knob:

- Rescue / medication: ρ≤0.5. Minimise t to maximise M(t); accept higher collision probability.

- Surveillance: ρ≥5.0. Late data retains value; collision yields M=0.

This reframes planner selection from “which algorithm wins?” to “what risk level does this mission warrant?”—a question FLARE makes empirically answerable.

## 4.3	Limitations

FLARE uses 2D grids without altitude or aerodynamics. The planners are classical; the CA fire model is approximate. Three Athens scenarios provide focused but not exhaustive coverage. Risk coefficients are fixed, not adapted online. The inversion is not obvious a priori: no prior benchmark includes time-coupled scoring, and |ΔRank|=3 is not predictable without experiments. Nor is it parameter tuning: ρ is a structural property of each planner’s cost function. The inversion persists across all three geographically distinct scenarios. These simplifications are deliberate: the contribution is the evaluation paradigm and the ranking-inversion finding, not the planners themselves.

# 5	Conclusions

We presented FLARE, a benchmark that answers a question conventional evaluations cannot pose: does the best navigator also rescue most effectively? Across 450 episodes on three Greek wildfire scenarios, the answer is no. The planner with the highest survival probability ranks last in M(t) because its detours increase t beyond the point where delivery retains value. A risk-tolerant planner achieves the highest mission score despite ranking third in navigation (Friedman p<0.001).

This finding—the ranking inversion—demonstrates that navigation success rate is not merely incomplete but actively misleading for time-critical operations: it rewards the wrong planner. The implication is a paradigm shift: from evaluating whether planners succeed to evaluating when they deliver. The risk coefficient provides the deployment knob; FLARE provides the evaluation platform. Both are open-source.

Code and data: https://github.com/zervakisai/uavplan.

## Author Contributions

Conceptualization, K.Z. and E.P.; methodology, K.Z.; software, K.Z.; validation, K.Z. and E.P.; formal analysis, K.Z.; investigation, K.Z.; resources, K.Z.; data curation, K.Z.; writing—original draft preparation, K.Z.; writing—review and editing, K.Z. and E.P.; visualization, K.Z.; supervision, E.P.; project administration, K.Z. All authors have read and agreed to the published version of the manuscript.

## Funding

This research received no external funding.

## Data Availability Statement

The benchmark code, scenario configurations, and full experimental results are publicly available at https://github.com/zervakisai/uavplan. Map data are derived from OpenStreetMap  under the Open Database License (ODbL).

## Acknowledgments

Map data © OpenStreetMap contributors (ODbL). Scenarios inspired by the 2021 Evia wildfire , 2023 Rhodes fires , and 2023 Evros megafire . Data from EFFIS  and Copernicus EMS .

## Conflicts of Interest

The authors declare no conflicts of interest.

## Abbreviations

The following abbreviations are used in this manuscript:

| UAV | Unmanned Aerial Vehicle |
| --- | --- |
| FLARE | Fire-Landscape Adaptive Risk Evaluation |
| SR | Success Rate |
| IQM | Interquartile Mean |
| CI | Confidence Interval |
| CA | Cellular Automaton |
| APF | Artificial Potential Field |
| OSM | OpenStreetMap |
| NFZ | No-Fly Zone |
| RNG | Random Number Generator |
| WUI | Wildland–Urban Interface |

10 url@samestyle

M. Erdelj, E. Natalizio, K. R. Chowdhury, and I. F. Akyildiz, “Help from the sky: Leveraging UAVs for disaster management,” IEEE Pervasive Computing, vol. 16, no. 1, pp. 24–32, 2017.

C. Yuan, Y. Zhang, and Z. Liu, “A survey on technologies for automatic forest fire monitoring, detection, and fighting using unmanned aerial vehicles and remote sensing techniques,” Canadian Journal of Forest Research, vol. 45, no. 7, pp. 783–792, 2015.

G. Yücesoy et al., “The role of drones in disaster response: A literature review of operations research applications,” International Transactions in Operational Research, 2025.

Z. Siddiqui et al., “A systematic literature review of drones in emergency medicine: Practical applications, legal challenges, and future directions,” Drone Systems and Applications, 2024.

A. Moustafa et al., “Advances in UAV path planning: A comprehensive review of methods, challenges, and future directions,” Drones, vol. 9, no. 5, p. 376, 2025.

A. Dradoum, A. Khelassi, and F. Lachekhab, “Intelligent path planning algorithms for UAVs: Classification, complexity analysis, hybrid ablation insights, and future directions,” Advances in Mechanical Engineering, 2025.

S. Aggarwal and N. Kumar, “Path planning techniques for unmanned aerial vehicles: A review, solutions, and challenges,” Computer Communications, vol. 149, pp. 270–299, 2020.

Y. Zhao, Z. Zheng, and Y. Liu, “Survey on computational-intelligence-based UAV path planning,” Knowledge-Based Systems, vol. 158, pp. 54–64, 2018.

P. Henderson, R. Islam, P. Bachman, J. Pineau, D. Precup, and D. Meger, “Deep reinforcement learning that matters,” in Proceedings of the 32nd AAAI Conference on Artificial Intelligence, 2018.

R. Agarwal, M. Schwarzer, P. S. Castro, A. Courville, and M. G. Bellemare, “Deep reinforcement learning at the edge of the statistical precipice,” in Advances in Neural Information Processing Systems, vol. 34, 2021.

A. Alexandridis, D. Vakalis, C. I. Siettos, and G. V. Bafas, “A cellular automata model for forest fire spread prediction: The case of the wildfire that swept through Spetses island in 1990,” Applied Mathematics and Computation, vol. 204, no. 1, pp. 191–201, 2008.

J. Demšar, “Statistical comparisons of classifiers over multiple data sets,” Journal of Machine Learning Research, vol. 7, pp. 1–30, 2006.

E. D. Dolan and J. J. Moré, “Benchmarking optimization software with performance profiles,” Mathematical Programming, vol. 91, no. 2, pp. 201–213, 2002.

M. Towers, A. Kwiatkowski, J. K. Terry, J. U. Balis, G. De Cola, T. Deleu, M. Goulão et al., “Gymnasium: A standard interface for reinforcement learning environments,” in Advances in Neural Information Processing Systems, vol. 38, 2025, spotlight.

G. Boeing, “OSMnx: New methods for acquiring, constructing, analyzing, and visualizing complex street networks,” Computers, Environment and Urban Systems, vol. 65, pp. 126–139, 2017.

P. E. Hart, N. J. Nilsson, and B. Raphael, “A formal basis for the heuristic determination of minimum cost paths,” IEEE Transactions on Systems Science and Cybernetics, vol. 4, no. 2, pp. 100–107, 1968.

S. Koenig and M. Likhachev, “D* Lite,” in Proceedings of the 18th AAAI Conference on Artificial Intelligence, 2002, pp. 476–483.

O. Khatib, “Real-time obstacle avoidance for manipulators and mobile robots,” The International Journal of Robotics Research, vol. 5, no. 1, pp. 90–98, 1986.

H. Rao et al., “Comprehensive review of path planning techniques for unmanned aerial vehicles (UAVs),” ACM Computing Surveys, 2025.

W. Zhou et al., “A risk-based unmanned aerial vehicle path planning scheme for complex air–ground environments,” Risk Analysis, 2025.

C. Melissaris et al., “Risk-aware UAV trajectory optimization using open urban GIS data and target level of safety constraints,” Drones, vol. 9, no. 10, p. 666, 2025.

F. Bonsignorio et al., “Towards a science of reproducible results in robotics,” Nature Machine Intelligence, 2025.

F. Giannino et al., “Statistical dynamics of wildfire burned area from cellular-automata simulators,” Scientific Reports, vol. 15, 2025.

M. Ferrara et al., “Patterns of burned area by a cellular-automata fire simulator: The role of microscale wind field,” Science China Earth Sciences, 2025.

L. P. Kaelbling, M. L. Littman, and A. R. Cassandra, “Planning and acting in partially observable stochastic domains,” Artificial Intelligence, vol. 101, no. 1–2, pp. 99–134, 1998.

M. Lauri, D. Hsu, and J. Pajarinen, “Partially observable Markov decision processes and robotics,” Annual Review of Control, Robotics, and Autonomous Systems, vol. 6, pp. 491–519, 2023.